WF-500 Coin Acceptor Operated Timer Control Board Selection Guide

1、Type：

| Type | Relay Channel | Save or Not | LED Bits | Power supply | Setup Button | Check Income History |
| --- | --- | --- | --- | --- | --- | --- |
| WF-500E | 1 | YES (Standard) | 4 | DC12V | 2 | NO |
| WF-500C | 4 | YES (Standard) | 4 | DC12V | 2 | NO |
| WF-500USB | Double USB | YSE (Standard) | 4 | DC12V | 2 | NO |
| WF-500G | 1 | YSE (Standard) | 4 | DC12V | 2 | NO |
| WF-500LCD | 1 | SWITCH | 4 | DC12V | 3 | YES |
| WF-502LCD | 2 | SWITCH | 4 | AC100V-240V | 3 | YES |

All type is user programmable vend price

Can be used with either a mechanical or electronic coin mechanism or pulse type bill acceptor

2、Selection

WF-500E: One relay output , payment value can be added when timer is running

Suitable for Simple type Power supply control or other one relay channel output control

WF-500C: Four relay Output channels are under controlled at the same time

Suitable for more seperate channels that need to be controlled at the same time

WF-500USB: Double USB channel are under controlled at the same time

Suitable for Single or double channel USB type control ,for example the game machine to control the USB

play knob.

WF-500G: One relay output , Payment value can not be added when timer is running, when timer start to run ,at

The same time,that power supply to coin acceptor is cut off.

WF-500LCD: One relay output ,payment value can be added when timer is running. And the onboard LCD

Display panel is used the setup the system parameters and also can check the income history.

WF-502LCD: Two relay output , and two output is controlled separately.

3、Customised Software

Everyone’s needs are different and even if your needs are not quite what we have described above, we can adapt our software to meet your needs